| BR 887 395 | PremiumLine frostprotected self-draining eye shower station with surrounding protection cage, with underfloor actuation |
| --- | --- |
| PremiumLine frostprotected self-draining eye shower station with surrounding protection cage, with underfloor actuation - frame made of stainless steel tubes, dimensions (H x W x D): 960 x 850 x 900 mm - platform actuation for eye shower made of galvanized steel, mounted with chain on angle diversion, dimensions (W x D): 800 x 800 mm - angle diversion made of stainless steel, with stainless steel attachment to the standpipe - ball valve made of stainless steel as underfloor actuation, mounted in stainless steel cabinet, with pull rod activation 1000 mm, water inlet ¾” male, DIN-DVGW tested and certificated, with automatic self-draining in 1000 mm depth - base plate with 4 mounting holes of stainless steel, polished, dimensions 200 x 200 mm - stand pipe 1 ¼” of stainless steel, polished, with bottom water inlet 1 ¼” female, total height 1025 mm - integrated PremiumLine safety eye shower with stainless steel bowl, mounting height 970 mm - connection pipe of stainless steel for easy installation and alignment of the shower, polished - bowl made of stainless steel, diameter 275 mm, polished, water outlet 1 ¼” male - high-performance spray heads made of stainless steel for large-scale dispersion of water, with plastic spray plate, largely free of calcification, with rubber sleeves and sealed dust caps with folding mechanism, mounted over distributor fork - integrated flow regulation 14 litre / minute for a standard-compliant jet pattern at a specified working range of 2.5 to 5 bar flow pressure - sign for eye shower according to EN ISO 7010 und ASR A1.3, self-adhesive PVC-film, 100 x 100 mm, viewing distance 10 metre - according to BGI/GUV-I 850-0, DIN 1988 and EN 1717 - according to ANSI Z358.1-2014 and EN 15154-2:2006 - DIN-DVGW tested and certificated Manufacturer: B-SAFETY or equal Article-No.: BR 887 395 Technical Specifications Minimum flow pressure: 2.5 bar Operating pressure: 2.5 to 5 bar Flow rate: 14 l/min Water inlet: ¾” male |  |