| BR 955 005 | ClassicLine frostprotected heated tank eyewash station with 60-litre-tank, in wall cabinet |
| --- | --- |
| ClassicLine frostprotected heated tank eyewash station with 60-litre-tank, in wall cabinet - for use when no water connection is available - wall cabinet of galvanized steel, chemical resistant green powder coated, dimensions (H x W x D): 660 x 620 x 600 mm - additional wall brackets for better stability of galvanised steel, chemical resistant green powder coated - isolation-package of PPB 20 g/l, black - heating by self-regulating trace tape, output 20 W/m, electrical supply 230 V - 50 Hz - outdoor thermostat and energy-saving-circuit (shutdown of trace tape at 14°C, switch on at 4°C) - tank of chemical-resistant plastic, green, with a tank capacity of 60 litres - tank is sufficient for approx. 15 minutes uninterrupted flushing time - water inlet via large screw cap, top of tank - easy and simple activation by dragging down the pull strip - softly radiating low-pressure spray heads, which supply both eyes with the rinsing liquid - sign for eye shower according to DIN EN ISO 7010 und ASR A1.3 mounted on the cabinet door, self-adhesive PVC-film, 150 x 150 mm, viewing distance 15 metre - dimensions (H x W x D): 660 x 620 x 600 mm - suitable for use in EX zones 1 & 2, all electrical parts explosion-proof - according to ANSI Z358.1-2014 and DIN EN 15154-4:2009 Manufacturer: B-SAFETY or equal Article-No.: BR 955 005 Technical Specifications Material: GFK-plastic, galvanized steel Color: green Dimensions (H x W): 660 x 620 mm Tank capacity: 60 litre Flush time: about 15 minutes Water filling: screw cap Electrical supply: 230 V - 50 Hz |  |